- Jones Road Bridge over Willeo Creek, Structure ID 121-5263-0

- Reconstruct existing catch basin with Ga DOT 1033-E at the northwest corner of the bridge. Remove and replace guard rail and posts as necessary as part of the catch basin replacement.

- Remove existing 12” C.M.P., (approximately 20 lin. ft) and replace pipe with new 18” concrete pipe. Remove existing headwall and replace it with new headwall for new 18” concrete pipe.

- Fill voids under and around abutments with flowable fill as directed in the field.

- Place stone rip rap, type 1, to stabilize slope, approximately 15’ wide, 5’ deep, and 30’ long. Fill any void with flowable fill as necessary as directed.

- Norcross Street Bridge over Hog Wallow Creek, Structure ID 121-0309-0

- Weld steel plate, ½” thick, 2’ wide, and 3’ long, to the inside web between the upper and lower flanges to the Northwest outside “I” beam covering the section loss on that beam.

- All disturbed surface related to this work shall be primed and coated with minimum of two coats of finish paint.

- Oxbo Road Bridge over Hog Wallow Creek, Structure ID 121-5134-0

- All work described in the followings are to be done as directed in the field. The dimensions are approximate; the field conditions will dictate the final dimensions and quantities.

- Install approximately 20 lin. ft., 5’ wide, of concrete sidewalk on South side of the bridge.

- Remove existing headwall and replace with a new manhole complete with cone section and ring and cover.

- Install a new 18” C.M.P. from new manhole to discharge storm run-off.

- Remove and replace existing curb and gutter, approximately 25 lin. ft. on the Southwest side of the bridge.

- Remove and reconstruct existing catch basin top and throat section of the catch basin. Remove and reset existing guard rail and posts as necessary.

- Mill existing asphaltic concrete over bridge deck to the depth of 1-1/2” to 2-1/2”, but not to exceed the thickness of existing asphalt over the bridge deck.