12.4.201  Neighborhood Residential District Requirements

| 12.4.210 Site Development Regulations | District Requirements |
| --- | --- |
| Building Use |  |
| Commercial | not permitted |
| Office | not permitted |
| Residential/Institutional | see Section 12.4.010, Building Use |
| 12.4.211 Building Height |  |
| Minimum height | none |
| Maximum height | 40 feet |
| 12.4.212 Maximum Building Length |  |
| Maximum | 300 feet |
| 12.4.213 Building Orientation |  |
| Required or not required | required |
| 12.4.214 Frontage Types |  |
| Storefront | not permitted |
| Forecourt | not permitted |
| Stoop | not permitted |
| Terraced Yard | permitted |
| Common Yard | permitted |
| 12.4.214 Transparency/Fenestration (Transparency zone: 30” to 8’ above grade) |  |
| Terraced Yard | 10% single family; 15% multi-family |
| Common Yard | none required |
| 12.4.215 Front Yard Setback (from back of sidewalk) |  |
| Terraced Yard |  |
| Single family uses | Min 6’; Max 12’ |
| Multi-family uses | Min 6’; Max 12’ |
| Common Yard | Min 6’; Max 12’ |
| 12.4.215 Side Yard Setback |  |
| Minimum | 7’ |
| 12.4.215 Rear Yard Setback |  |
| Minimum | 20’ |
| 12.4.210 Site Development Regulations | District Requirements |
| 12.4.216.1 Building Articulation Features |  |
| Residential buildings | Every 30’ |
| 12.4.216.2 Maximum Façade Width Checklist |  |
| Required or not required | required for facades wider than 100’ |
| Large-scale Retail Use Standards |  |
| Minimum | N/A |
| 12.4.216.3 Building Materials |  |
| Wood siding | permitted; above 2 ft above grade |
| Concrete siding | permitted; above 2 ft above grade |
| Stucco | permitted; above 2 ft above grade |
| Stone | permitted |
| Brick, precast concrete and timber frame | permitted |
| 12.4.216.4 Multi-family Building Details |  |
| Required or not required | required |
| 12.4.216.5 Blank Wall Treatment |  |
| Required or not required | required |
| 12.4.217 High Visibility Street Corner Standards |  |
| Required or not required | required for buildings at designated sites, see Community Map |
| 12.4.220 Landscape and Streetscape Regulations | District Requirements |
| Open Space Requirement |  |
| Minimum | N/A |
| 12.4.221 Landscaping Standards |  |
| Required or not required | required |
| 12.4.221 Parking Lot Landscape Standards |  |
| Required or not required | required |
| 12.4.222 Utilities Area Screening |  |
| Required or not required | required |
| 12.4.223 Sidewalk Standards |  |
| Minimum | Refer to Table 12.3.5 of the Roswell Zoning Ordinance |
| Outdoor dining | N/A |
| Curb cuts | Minimum distance between: 25’; no cuts within 100’ of intersection |
| 12.4.230 Parking Regulations | District Requirements |
| 12.4.231 Parking Standards |  |
| Residential, multi-family | Min 1 space/unit; Max 1.5 spaces/unit |
| Residential, single family | Min 1 space/unit; Max 3 spaces/unit |
| 12.4.232 Maximum Parking Lot Frontage |  |
| Maximum | 50% of lot width |
| 12.4.233 Parking Structure Design Standards |  |
| Required or not required | required |
| 12.4.234 Alleys |  |
| Minimum width | 20’ |
| 12.4.240 Lighting and Signage Regulations | District Requirements |
| 12.4.241 Lighting Standards |  |
| Required or not required | required |
| 12.4.242 Signage Standards |  |
| Required or not required | required |