**This is what went to Committee after the First Reading**

STATE OF GEORGIA 				First Reading: September 23, 2013

COUNTY OF FULTON 				Second Reading: September 30, 2013

AN ORDINANCE TO AMEND CHAPTER 12 OF THE CITY OF ROSWELL ZONING

ORDINANCE REGARDING BUILDING MATERIAL ALLOWED AS PART OF NEW

DEVELOPMENT

WHEREAS, the City of Roswell has amend its Zoning Ordinances to provide specific

provisions for the Groveway Community; and

WHEREAS, the Mayor & City Council have determined that the new regulations for the Groveway Community are to protect the health, safety and welfare of the residents of Roswell; and

WHEREAS, the Mayor & City Council have been part of the charrette planning process completed in the Groveway Community; and

WHEREAS, the Mayor & City Council have determined that an amendment to the

Zoning Ordinance related to allowable building material in Groveway is needed: and

NOW, THEREFORE, pursuant to their authority, The Mayor & City Council adopt the following ordinance:

1.

Chapter 12, of the Zoning Ordinances is hereby amended by changes to Section 12.4.19,

to read as follows:

12.4.19 Building Materials

The following standards shall be utilized for high-quality development and the use of

building materials that will be consistent with and add to the character of the area.

Brick and concrete siding standards

Masonry, concrete, brick or other durable material must be incorporated between wood

trim and the ground plane (at least 2 feet above grade). Brick may also be used as a

primary façade material.

Stucco standards

Proper trimming. Stucco and similar troweled finishes (excluding Exterior Insulation and

Finish System or “EIFS”) must be sheltered from extreme weather and are limited to no

more than 50 percent of façades containing a customer or resident entry.

Treatment near ground level. Stucco and similar surfaces should not extend below 2 feet

above the ground plane. Concrete, masonry, brick or other durable material must be used

below this line.

Stone and timber standards

When used for the primary façade (containing the primary pedestrian entrance), buildings

are encouraged to incorporate a combination of textures and/or colors. For example,

stone, masonry, brick or timber applied as columns or piers combined with wood trimming,

concrete siding or stucco may be appropriate.

Prohibited materials

The following materials are prohibited: mirrored glass, chain-link fencing and back-lit vinyl awnings.

Inappropriate materials

The following materials are inappropriate for use in Groveway Community Overlay District and shall not be approved, except upon a showing of one or more of the criteria set out in Section 31.4.8 of the Zoning Ordinance. Expense alone, absent one or more of the criteria set out in Section 31.4.8, shall not be sufficient to justify use of an inappropriate material.

Vinyl or vinyl clad windows, and windows with removable muntins (“grilles”) or

muntins sandwiched between the glass; back-lit vinyl awnings; metal siding, including vinyl or aluminum; fiber cement siding with a wood grain surface.

Wood or Aluminum-Clad Wood Windows are appropriate for new development projects. The use of fiber cement is appropriate for new development projects provided it has a smooth surface texture.

2.

All ordinances, parts of ordinances, or regulations in conflict herewith are repealed.

3.

Severability. Should any court of competent jurisdiction declare any section of this Ordinance invalid or unconstitutional, such declaration shall not affect the validity of the Ordinance as a whole or any part thereof, which is not specifically declared to be invalid or unconstitutional.

4.

It is the intention of the Mayor and Council, and it is hereby ordained that the provisions of this Ordinance shall become and be made a part of the City of Roswell Zoning Ordinance, Georgia and the sections of this Ordinance may be renumbered to accomplish such intention.

5.

This Ordinance shall take effect and be in force from and after the date of ___________, 2013, the public welfare demanding it.

6.

The above Ordinance was read and approved by the Mayor and Council of the City of Roswell,

Georgia on the _____ day of _________ , 2013.

_____________________________

Jere Wood, Mayor

- Attest:

__________________________

- Marlee Press, City Clerk

- SEAL

- __

- Councilmember Jerry Orlans				Councilmember Rebecca Wynn

- ____________________								__

- Councilmember Betty Price				Councilmember Richard Dippolito

- __

- Councilmember Kent Igleheart				Councilmember Nancy Diamond